/*****************************************************

Project : SMS Sender

Version : 1.0

Date    : 1392/07/20

Author  : Ali Imanifar

Company : ECA

Comments: This is a sample program for SIM900 Learning board produce by ECA

www.ECA.ir

www.Eshop.ECA.ir

www.ECA.ir/Forum2

Chip type               : ATmega32

Program type            : Application

AVR Core Clock frequency: 11.059200 MHz

Memory model            : Small

External RAM size       : 0

Data Stack size         : 512

*****************************************************/

#include <mega32.h>

#include <stdlib.h>

#include <delay.h>

#include <keypad.h>

#include <SIM900.h>

// Alphanumeric LCD functions

#include <alcd.h>

// Declare your global variables here

char key=16;

char sms[256],phone[30];

int i=0;

// Standard Input/Output functions

#include <stdio.h>

void main(void)

{

// Declare your local variables here

// Input/Output Ports initialization

// Port A initialization

// Func7=In Func6=In Func5=In Func4=In Func3=In Func2=In Func1=In Func0=In

// State7=T State6=T State5=T State4=T State3=T State2=T State1=T State0=T

PORTA=0x00;

DDRA=0x00;

// Port B initialization

// Func7=In Func6=In Func5=In Func4=In Func3=In Func2=In Func1=In Func0=In

// State7=T State6=T State5=T State4=T State3=T State2=T State1=T State0=T

PORTB=0x00;

DDRB=0x00;

// Port C initialization

// Func7=In Func6=In Func5=In Func4=In Func3=Out Func2=Out Func1=Out Func0=Out

// State7=T State6=T State5=T State4=T State3=0 State2=0 State1=0 State0=0

PORTC=0x00;

DDRC=0x0F;

// Port D initialization

// Func7=In Func6=Out Func5=Out Func4=In Func3=In Func2=In Func1=In Func0=In

// State7=T State6=0 State5=0 State4=T State3=T State2=T State1=T State0=T

PORTD=0x00;

DDRD=0x60;

// USART initialization

// Communication Parameters: 8 Data, 1 Stop, No Parity

// USART Receiver: On

// USART Transmitter: On

// USART Mode: Asynchronous

// USART Baud Rate: 9600

UCSRA=0x00;

UCSRB=0x18;

UCSRC=0x86;

UBRRH=0x00;

UBRRL=0x47;

// Alphanumeric LCD initialization

// Connections are specified in the

// Project|Configure|C Compiler|Libraries|Alphanumeric LCD menu:

// RS - PORTA Bit 0

// RD - PORTA Bit 1

// EN - PORTA Bit 2

// D4 - PORTA Bit 4

// D5 - PORTA Bit 5

// D6 - PORTA Bit 6

// D7 - PORTA Bit 7

// Characters/line: 16

lcd_init(16);

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Only GOD");

lcd_gotoxy(0,1);

lcd_puts("www.ECA.ir");

delay_ms(3000);

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Power ON The");

lcd_gotoxy(0,1);

lcd_puts("SIM900? Y:1  N:2");

i = 17;

do

i = keyscan();

while ((i != 1) & (i != 2));

if(i == 1) {

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Powering ON ...");

SIM900_PWR_ON();

lcd_gotoxy(0,1);

lcd_puts("Powered ON");

}

else {

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Note:");

lcd_show("You choose to powering on the SIM900 by yuor self so if the module is off please use the power button on the module to power it on",1);

while(SIM900_Is_ON() == 0)

{

delay_ms(300);

lcd_gotoxy(0,1);

lcd_puts("Power ON SIM900 ");

}

delay_ms(2000);

}

delay_ms(1000);

//++++++++++++++++++++++++++++++++++++++++++++++

i = SIM900_Init();

lcd_clear();

if(i==1)

{

lcd_gotoxy(0,0);

lcd_puts("Sim900 Init");

}

else

{

lcd_gotoxy(0,0);

lcd_puts("Not Init");

}

//++++++++++++++++++++++++++++++++++++++++++++++

i = Net_Is_Register();

if(i==1)

{

lcd_gotoxy(0,1);

lcd_puts("Register Network");

}

else

{

lcd_gotoxy(0,1);

lcd_puts("Not Register");

}

//++++++++++++++++++++++++++++++++++++++++++++++

delay_ms(1000);

// Global enable interrupts

#asm("sei")

while (1)

{

// Place your code here

//delay_ms(10);

//lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Press 1 to send");

lcd_gotoxy(0,1);

lcd_puts("SMS             ");

key = keyscan();

if(key == 1)

{

delay_ms(200);

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Phone Number :");

if(number_keyscan(phone) == 1)

{

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("SMS Context :");

if(mobile_keyscan(sms) == 1)

{

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Sending ...");

SMS_Send(sms,phone);

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("SMS Sent");

delay_ms(1500);

}

else

{

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Canceled!");

delay_ms(1500);

}

}

else

{

lcd_clear();

lcd_gotoxy(0,0);

lcd_puts("Canceled!");

delay_ms(1500);

}

}

}

}